| ## NBBY | Ningbo Boyi Electron co.,ltd. |  |  | Product Specification Device Part No:BY-HUIR-5102B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Features： - Lens Type：Blue Transparent - Device Outline：Φ5mm Round Type |  |  |  |  |  |  |  |  |  |  |  |
| Absolute Maximim Ratings （Ta = 25℃） |  |  |  |  |  |  |  |  |  |  |  |
| Series |  | Symbol | Testing Condition |  |  | Characteristics |  |  |  | Unit |  |
|  |  |  |  |  |  | Max. |  | Min. |  |  |  |
| Power Dissipation |  | Pd | — |  |  | 150 |  | — |  | mW |  |
| Reverse Voltage |  | VR | IR = 10μA |  |  | — |  | 10 |  | V |  |
| Peak Forward Current |  | Ipeak | Duty=0.1，1kHz |  |  | 1000 |  | — |  | mA |  |
| Operating Temperature Range |  | Topr | — |  |  | +80 |  | -40 |  | ℃ |  |
| Storage Temperature Range |  | Tstr | — |  |  | +100 |  | -40 |  | ℃ |  |
| Lead Soldering Temperature |  | Tsol | 260℃ for 5sec.（3.0mm from resin） |  |  |  |  |  |  |  |  |
| Electro-Optical Characteristics （Ta = 25℃） |  |  |  |  |  |  |  |  |  |  |  |
| Series |  | Symbol | Testing Condition |  | Characteristics |  |  |  |  |  | Unit |
|  |  |  |  |  | Max. |  | Typ. |  | Min. |  |  |
| Forward Voltage |  | VF | IF = 50mA |  | 1.5 |  | 1.30 |  | 1.1 |  | V |
| Reverse Current |  | IR | VR = 5V |  | 10 |  | — |  | — |  | μA |
| Peak Sensing Wavelength |  | λp | IF =50mA |  | — |  | 940 |  | — |  | nm |
| Spectrum Width of Half Value |  | Δλ | IF = 20mA |  | — |  | 50 |  | —— |  | nm |
| 50%RadiantPower Angle |  | 2ф1/2 | IF = 50mA |  | — |  | 30 |  |  |  | ° |
| Radiantion Power |  | Pr | IF = 50mA |  | — |  | 7.0 |  | — |  | Mw/sr |
| Remarks： Radiant Intensity is measured by JF-Ⅱ tester of BOYI |  |  |  |  |  |  |  |  |  |  |  |